8086     Microprocessor     Addressing Modes

What is Addressing Mode? Addressing  modes  are  different  ways  by  which  CPU  can  access  data  or operands. They determine how to access a specific memory address. To load any data from and to memory/registers, MOV instruction is used.

The syntax of MOV instruction is:

MOV Destination, Source

It copies the data of 2nd operand (source) into the 1st operand (destination). To access  memory,  segment  registers  are  used  along  with  general-purpose registers.

- Register addressing mode

This  mode  involves  the  use  of  registers.  These  registers  hold  the  operands. This mode is very fast as compared to others because CPU doesn’t need to access memory. CPU can directly perform an operation through registers.

For example:

MOV AX,BL

MOV AL,BL

The above two instructions copy the data of BL register to AX and AL.

- Immediate Addressing Mode

In  this  mode,  there  are  two  operands.  One  is  a  register  and  the  other  is  a constant value. The register comes quickly after the op code.

For example:

The instruction

MOV AX, 30H

copies hexadecimal value 30H to register AX.

The instructions

MOV BX, 255

copies decimal value 255 to register BX.

You cannot use the immediate addressing mode to load immediate value into segment  registers.  To  move  any  value  into  segment  registers,  first  load  that value into a general-purpose register then add this value into segment register.

- Direct Addressing Mode

It  loads  or  stores  the  data  from  memory  to  register  and  vice  versa.  The instruction  consists  of  a  register  and  an  offset  address.

MOV CX,[481]

The  hexadecimal  value  of  481  is  1E1.  Assume  DS=2162H  then  the  logical address  will  be  2162:01E1.

- Register Indirect Addressing Mode

The register indirect addressing mode uses the offset address which resides in one of these three registers i.e., BX, SI, DI. The sum of offset address and the DS value shifted by one position generates a physical address.

For example:

MOV AL, [SI]

This instruction will calculate the physical address by shifting DS to the left by one  position  and  adding  it  to  the  offset  address  residing  in  SI.  The  brackets around SI indicates that the SI contain the offset address of memory location whose data needs to be accessed. If brackets are absent, then the instruction will   copy   the   contents   of   SI   register   to   AL.   Therefore,   brackets   are necessary

- Based Relative Addressing Mode

This addressing mode uses a base register either BX or BP and a displacement value to calculate physical address.

Physical Address= Segment Register (Shifted to left by 1) + Effective address

The effective address is the sum of offset register and displacement value.

The default segments for BX and BP are DS and SS.

For example:

MOV[BX+5],DX

In this example, the effective address is BX + 5 and the physical address is DS (shifted left) + BX+5.The instruction on execution will copy the value of DX to memory location of physical address= DS (shifted left) +BX+5.

- Indexed Relative Addressing Mode

This addressing mode is same as the based relative addressing mode.

The only difference is it uses DI and SI registers instead of BX and BP registers.

For example:

Given that

DS=704, SI = 2B2,

DI= 145MOV[DI]+12,AL

MOV[DI]+12,AL

This  instruction  on  execution  will  copy the  content  of  AL  at  memory  address 7197 (7040 + 145 + 12)

MOVBX,[SI]+10 This instruction will load the contents from memory address 7302 (7040 +2B2 +10) to register BX.

- Based Indexed Addressing Mode

The based indexed addressing mode is actually a combination of based relative addressing  mode  and  indexed  relative  addressing  mode.  It  uses  one  base register (BX, BP) and one index register (SI, DI).

For example:

MOVAX,[BX+SI+20]

The above instruction can also be written as:

MOVAX,[SI+BX+20] Or MOVAX,[SI][BX]+20

In this case, the physical address will be DS (Shifted left) + SI + BX + 20. Now, if we replace BX with BP then the physical address is equal to SS (Shifted left) + SI + BX + 20.

INSTRUCTIONS IN 8086

PIN DIAGRAM 8086

Interrupts in 8086

8086 Interrupt Types:

The8086 Interrupt Types are: Dedicated and Software interrupts

Dedicated Interrupts:

Type 0 : Divide by Zero Interrupt

When the quotient from either a DIV or IDIV instruction is too large to fit in the  result  register;  8086  Interrupt  Types  will  automatically  execute  type  0 interrupt.

Type 1 : Single Step Interrupt

The type 1 interrupt is the single step trap. In the single step mode, system will  execute  one  instruction  and  wait  for  further  direction  from  user.  Then user can examine the contents of registers and memory locations and if they are  correct,  user  can  tell  the  system  to  execute  the  next  instruction.  This feature is useful for debugging assembly language programs.

An 8086 Interrupt Types system is used in the single step mode by setting the trap flag. If the trap flag is set, the 8086 will automatically execute a type 1  interrupt  after  execution  of  each  instruction. But  the  8086  has  no  such instruction  to  directly  set  or  reset  the  trap  flag.  These  operations  can  be performed  by  taking  the  flag  register  contents  into  memory,  changing  the memory contents so to set or reset trap flag and save the memory contents into flag register.

Type 2 : Non Maskable Interrupt

As  the  name  suggests,  this  interrupt  cannot  be  disabled  by  any  software instruction. This interrupt is activated by low to high transition on 8086 NMI input pin. In response, 8086 will do a type 2 interrupt.

Type 3 : Breakpoint

The  type  3  interrupt  is  used  to  implement break  point function  in  the system.

The  type  3  interrupt  is  produced  by  execution  of  the  INT  3 instruction.

Break point function is often used as a debugging aid in cases where single stepping provides more detail than wanted.

When you insert a breakpoint,  the  system  executes  the  instructions  upto  the  breakpoint,  and then goes to the breakpoint procedure.

In the break point procedure you can write  a  program  to  display  register  contents,  memory  contents  and  other information that is required to debug your program.

You can insert as many breakpoints as you want in your program.

Type 4 : Overflow Interrupt

The  type  4  interrupt  is  used  to  check  overflow  condition  after  any  signed arithmetic operation in the system. The 8086 Interrupt Types overflow flag, OF, will be represented in the destination register or memory location.

For example, if you add the 8-bit signed number 0111 1000 (+ 120 decimal) and the 8 bit signed number 0110 1010 (t 106 decimal), result is 1110 0010 (-98 decimal).

In signed numbers, MSB (Most significant Bit) is reserved for sign  and  other  bits  represent magnitude  of  the  number.  In  the  previous example, after addition of two 8-bit signed numbers result is negative, since it is too large to fit in 7 bits. To detect this condition in the program, you can put interrupt on overflow instruction, INTO, immediately after the arithmetic instruction  in  the  program.  If  the  overflow  flag  is  not  set  when  the  8086 executes the INTO instruction, the instruction will simply function as an NOP (no  operation).  However,  if  the  overflow  flag  is  set,  indicating  an  overflow error,  the  8086  will  execute  a  type  4  interrupt  after  executing  the  INTO instruction.

Another way to detect and respond to an overflow error in a program is to put  the  jump  if  overflow  instruction  (JO)  immediately  after  the  arithmetic instruction.  If  the  overflow  flag  is  set  as  a  result  of  arithmetic  operation, execution  will  jump  to  the  address  specified  in  the  JO  instruction.  At  this address, you can put an error routine which responds in the way you want to the overflow.

Software Interrupts:

Type 0 –255 :

The 8086 INT instruction can be used to cause the 8086 to do one of the 256 possible interrupt types. The interrupt type is specified by the number as a part of the instruction. You can use an INT2 instruction to send execution to an  NMI  interrupt  service routine.  This  allows  you  to  test  the  NMI  routine without  needing  to  apply  an  external  signal  to  the  NMI  input  of  the  8086 Interrupt Types.

Macros and Procedures in 8086

Procedure:

In a program, we very frequently face situations where there is a need to perform the same set of task again and again. So, for that instead of writing the same sequence of instructions,  again  and  again,  they  are  written  separately  in  a  subprogram.  This subprogram  is  called  a  procedure.  With  the  help  of  procedures,  we  can  very  well implement the concept of modular programming in our code. Also, whenever we need to execute the instructions mentioned in the procedure, we can simply make a CALL to it. Therefore, with the help of procedures, the duplicity  in the instructions can be avoided.

The Syntax for a procedure is as follows:

•Here, the PROC is a keyword to define that the set of instructions enclosed by the given name is a procedure.

•The ENDP keyword defines that the body of the procedure has been ended.

•All the instructions lying between these two keywords are the instructions that belong  to  the  procedure  and  will  be  executed  whenever  a CALL to  the procedure is made.

•The keyword near or far defines the range of code within which the procedure is  defined.  If  it  is  defined  in  the  same  segment  as  the  rest  code,  then  near  is used.

•If it is defined in some other segment, then the keyword far is used for it.

At the end of the procedure, the RET instruction is used. This instruction will cause the execution to be transferred to the program from which the call to the procedure was made.

Macro:

A Macro is a set of instructions grouped under a single unit. It is another method for implementing modular programming in the 8086 microprocessors (The first one was using Procedures).

The Macro is different from the Procedure in a way that unlike calling and returning the control as in procedures, the processor generates the code in the program every time whenever and wherever a call to the Macro is made.

A Macro can    be    defined    in    a    program    using    the    following    assembler directives: MACRO(used  after  the  name  of  Macro  before  starting  the  body  of  the Macro) and ENDM(at the end of the Macro).

All the instructions that belong to the Macro lie within these two assembler directives.

The following is the syntax for defining a Macro in the 8086 Microprocessor:

It is optional to pass the parameters in the Macro. If you want to pass them to your macros, you can simply mention them all in the very first statement of the Macro just after the directive: MACRO.

The advantage of using Macro is that it avoids the overhead time involved in calling and returning (as in the procedures). Therefore, the execution of Macros is faster as compared  to  procedures.  Another  advantage  is  that  there  is  no  need  for  accessing stack  or  providing  any  separate  memory  to  it  for  storing  and returning  the  address locations while shifting the processor controls in the program.

But  it  should  be  noted  that  every  time  you  call  a  macro,  the  assembler  of  the microprocessor  places  the  entire  set  of  Macro  instructions  in  the  mainline  program from where the call to Macro is being made. This is known as Macro expansion. Due to  this,  the  program  code  (which  uses  Macros)  takes  more  memory  space  than  the code  which  uses  procedures  for  implementing  the  same  task  using  the  same  set  of instructions. Hence, it is better to use Macros where we have small instruction sets containing less number of instructions to execute.

Addressing modes of 8051

In 8051 There are six types of addressing modes.

•Immediate Addressing Mode

•Register Addressing Mode

•Direct Addressing Mode

•Register Indirect Addressing Mode

•Indexed Addressing Mode

•Implied Addressing Mode

1.Immediate addressing mode

In this Immediate Addressing Mode, the data is provided in the instruction itself. The data  is  provided  immediately  after  the  opcode.

These  are  some  examples  of Immediate Addressing Mode.

MOVA,#0AFH;

MOVDPTR,#FE00H;

In these instructions, the # symbol is used for immediate data. In the last instruction, there is DPTR. The DPTR stands for Data Pointer. Using this, it points the external data memory location. In the first instruction, the immediate data is AFH, but one 0 is added at the beginning. So when the data is starting with A to F, the data should be preceded by 0.

2. Direct Addressing Mode

In  the  Direct  Addressing  Mode,  the  source  or  destination  address  is  specified  by using 8-bit data in the instruction. Only the internal data memory can be used in this mode.

Here some of the examples of direct Addressing Mode.

MOV80H,R6;

MOVR2,45H;

MOVR0,05H;

The first instruction will send the content of registerR6 to port P0 (Address of Port 0 is 80H).

The  second  one  is  for getting  content  from  45H  to  R2.

The  third  one  is used to get data from Register R5 (When register bank RB0 is selected) to registerR5.

4.Register indirect addressing Mode

In  this  mode,  the  source  or  destination  address  is  given  in  the  register.  By  using register  indirect  addressing  mode,  the  internal  or  external  addresses  can  be accessed. The R0 and R1 are used for 8-bit addresses, and DPTR is used for 16-bit  addresses,  no  other  registers  can  be  used  for  addressing  purposes.

Let  us  see some examples of this mode.

MOV0E5H,@R0;

In  the  instructions,  the  @  symbol  is  used  for  register  indirect  addressing.  In  the instruction, it is showing that the R0 register is used. If the content of R0 is 40H, then that  instruction  will  take  the  data  which  is  located  at  location  40H  of  the  internal RAM.

5.Indexed addressing mode

In  the  indexed  addressing  mode,  the  source  memory  can  only  be  accessed  from program  memory. The  destination  operand  is  always  the  register  A.

These  are some examples of Indexed addressing mode.

MOVA,@A+PC;

MOVA,@A+DPTR;

For the first instruction, let us consider A holds 30H. And the PC value is 1125H. The  contents  of  program  memory  location 1155H  (30H  +  1125H)  are  moved  to register A.

6.Implied Addressing Mode

In  the  implied  addressing  mode,  there  will  be  a  single  operand.  These  types  of instruction can work on specific registers only. These types of instructions are also known   as   register   specific   instruction.

Here   are   some   examples   of   Implied Addressing Mode.

RLA;

SWAPA;

These are 1-byte instruction. The first one is used to rotate the A register content to the Left. The second one is used to swap the nibbles in A.

8051 ARCHITECTURE

8051 microcontroller is designed by Intel in 1981. It is an 8-bit microcontroller. It is built with 40 pins DIP (dual inline package), 4kb of ROM storage and 128bytes of RAM storage, 2 16-bit timers. It   consists   of four   parallel   8-bit   ports,   which   are   programmable   as   well   as addressable as per the requirement. An on-chip crystal oscillator is integrated in the microcontroller having crystal frequency of 12 MHz. Let us now discuss the architecture of 8051 Microcontroller. In the following diagram, the system bus connects all the support devices to the CPU. The system bus consists of an 8-bit data bus, a 16-bit address bus and bus control signals. All other devices like program memory, ports, data memory, serial interface, interrupt control, timers, and the CPU are all interfaced together through the system bus.

8051 Instruction Set

the 8051 Microcontroller Instruction Set is optimized for 8-bit control applications.

8051 Microcontroller instruction set can have up to 28 = 256 Instructions.

There are 49 Instruction Mnemonics in the 8051 Microcontroller Instruction Set and these 49 Mnemonics are divided into five groups:

DATA TRANSFER

ARITHMETIC

LOGICAL

BOOLEAN

PROGRAM BRANCHING

DATA TRANSFER:

MOV A, Rn - Moves the Rn register to the accumulator

The instruction moves the Rn register to the accumulator.

The Rn register is not affected.

Syntax: MOV A,Rn

Before execution:

R3=58h

After execution:

R3 = 58h A=58h

MOV A, #data - Moves the immediate data to the accumulator

Instruction moves the immediate data to the accumulator.

Syntax: MOV A, #28

After execution: A = 28h

MOV A, @Ri - Moves the indirect RAM to the accumulator

Instruction moves the indirectly addressed register of RAM to the accumulator.

The register address is stored in the Ri register (R0 or R1).

The result is stored in the accumulator.

The register is not affected.

Syntax: MOV A,@Ri

Register Address SUM=F2h R0=F2h

Before execution: SUM=58h

After execution: A=58h SUM=58h

Arithmetic

- ADD A, Rn; Adds the register Rn to the accumulator

Description: Instruction adds the register Rn (R0-R7) to the accumulator.

After addition, the result is stored in the accumulator

Before execution:

A=2Eh R4=12h

After execution:

A=40h R4=12h

- ADD A, #DATA

Data: constant within 0-255 (0-FFh)

Description: Instruction  adds  data  (0-255)  to  the  accumulator.  After  addition,  the  result  is stored in the accumulator.

ADD A, #33h

Before execution: A= 16h

After execution: A= 49h

- INC A - Increments the accumulator by1

Description: This instruction increments the value in the accumulator by 1.

If the accumulator includes the number 255, the result of the operation will be 0.

Before execution: A=E4h

After execution: A=E5h 6.

- DEC A -Decrements the accumulator by 1

Description: Instruction decrements the value in the accumulator by 1.

If there is a 0 in the accumulator, the result of the operation is FFh. (255 dec.)

Syntax: DEC A;

Byte: 1 (instruction code);

- DIV AB  -Divides the accumulator by the register B

Description: Instruction divides  the  value  in  the  accumulator  by  the  value  in  the  B  register.

After  division  the  integer  part  of  result  is  stored  in  the  accumulator  while  the register contains the remainder.

In case of dividing by 1, the flag OV is set and the  result  of  division  is  unpredictable.

The 8-bit quotient  is  stored  in  the accumulator and the 8-bit remainder is stored in the B register.

Before execution:

A=FBh (251dec.) B=12h (18 dec.)

After execution:

A=0Dh (13dec.) B=11h (17dec.)

13·18 + 17 =251

- MUL A B -Multiplies A and B

Description: Instruction multiplies the value in the accumulator with the value in  the  B  register.

The  low-order  byte  of  the  16-bit  result  is  stored  in  the accumulator, while the high byte remains in the B register.

If the result is larger than 255, the overflow flag is set.  The carry flag is not affected.

Before execution:

A=80 (50h)B=160 (A0h)

After execution:

A=0 B=32h A·B=80·160=12800 (3200h)

LOGICAL

SWAP A - Swaps nibbles within the accumulator

A: accumulator

A nibble refers to a group of 4 bits within one register (bit0-bit3 and bit4-bit7). This instruction interchanges high and low nibbles of the accumulator.

Syntax: SWAPA

Before execution: A=E1h (11100001)bin.

After execution: A=1Eh (00011110)bin.

ANL A,Rn  - AND register to the accumulator

A: accumulator

Rn: any R register(R0-R7)

Instruction performs logic AND operation between the accumulator and Rn register.

The result is stored in the accumulator.

Syntax: ANL A,Rn

Before execution: A= C3h (11000011 Bin.) R5= 55h (01010101 Bin.)

After execution: A= 41h (01000001 Bin.)

ORL A,Rn - OR register to the accumulator

Rn: any R register (R0-R7)

Instruction performs logic OR operation between the accumulator and Rn register.

The result is stored in the accumulator.

Syntax: ORLA,Rn

Before execution: A= C3h (11000011 Bin.) R5= 55h(01010101 Bin.)

After execution: A= D7h (11010111 Bin.)

CLR A - Clears the accumulator

A: accumulator

Instruction clears the accumulator.

Syntax: CLR A

After execution: A=0

Features and Architecture of Pentium Processor

Following are the features that Pentium processor offers:

Some of the features of Pentium architecture are:

Complex Instruction Set Computer (CISC) architecture with Reduced Instruction Set Computer (RISC) performance.

64-Bit Bus

Upward code compatibility.

Pentium processor uses Superscalar architecture and hence can issue multiple instructions per     cycle.

Multiple Instruction Issue (MII) capability.

Pentium processor executes instructions in five stages. This staging, or pipelining, allows the processor to overlap multiple instructions so that it takes less time to execute two instructions in a row.

The Pentium processor fetches the branch target instruction before it executes the branch instruction.

The Pentium processor has two separate 8-kilobyte (KB) caches on chip, one for instructions and one for data. It allows the Pentium processor to fetch data and instructions from the cache simultaneously.

When data is modified, only the data in the cache is changed. Memory data is changed only when the Pentium processor replaces the modified data in the cache with a different set of data

The Pentium processor has been optimized to run critical instructions in fewer clock cycles than the 80486 processor.

We have already mentioned in the beginning that Pentium is a superscalar processor. So, let us first understand,

What are Superscalar Processors?

A special category of microprocessors that involves a parallel approach for instruction execution called instruction-level parallelism through which more   than   one   instruction   gets   executed   in   one   clock   cycle   is called superscalar processors.  It  is  famous  as  a  second-generation RISC processor because RISC is the ones that operate in a faster manner with reduced instruction sets. Unlike  scalar  processors  that  have  the  ability  to  execute  maximal  one instruction per clock cycle, the superscalar processor uses the approach of  simultaneously  executing  two  instructions  in  one  clock  cycle.  The superscalar processors perform this task by sending multiple instructions to  various  execution  units  at  the  same  time.  Hence  this  provides  high throughput. It is to be noted here that superscalar processors are generally pipelined. However,   pipelining   is   different   from   super   scaling   in   a   way   that

superscalars  allow  execution  of  multiple  instructions  parallelly  using multiple execution units while pipelining uses a single execution unit which is  divided  into  multiple  phases  in  order  to  execute  multiple  instructions

Architecture of Pentium Microprocessor

The various functional units are as follows:

1.Bus unit

2.Paging unit

3.Control ROM

4.Prefetch buffer

5.Execution unit with two integer pipeline (U-pipe and V-pipe)

6.Code cache

7.Data cache

8.Instruction decode

9.Branch target buffer

10.Dual processing logic

11.Advanced programmable interrupt controller

Let us now understand, how the architectural operation takes place. The bus unit of the architecture sends the control signal and fetches cade and data from external memory and IO devices. The size of the external data  bus  is  64-bit  through  which  burst  read  and  burst  write-back  cycles can  be  achieved.  The  paging  unit  in  the  architecture  provides  optional extensions of around 2 to 4 Mb page sizes. In order to load the instructions into the execution unit, code cache, branch target buffer and prefetch buffers operate together. The code cache or the external  memory  holds  the  instructions  from  where  these  are  fetched. While the branch target buffer holds the address of the respective branch and  the  TLB  (translational  lookaside  buffer)  within  the  code  cache converts the linear address into the physical address that is used by the code cache. This processor contains pairs of prefetch buffers having a size of 32-byte that  combinedly  operate  with  branch  target  buffer.  Both  the  buffers operate  independently  but  not  at the  same  time.  One  of  the  prefetch buffers starts fetching the instructions in a sequential manner till the time branch  instruction  has  not  occurred.  However,  as  soon  as  the  branch instruction  is  fetched  by  the  prefetch  buffer  then  BTB  will  check  for  the branch but once it is checked by BTB that branch has not occurred then linear fetching of instruction will continue. On the contrary, while checking if BTB gets to know about the occurrence of the branch then the other prefetch buffer in pair gets enabled and starts fetching the instructions from the branch target address. By doing so, the branching  instructions  get  simultaneously  fetched  and  are  ready  for decoding and execution. The  execution  unit  within  the  Pentium  microprocessor  contains  two integer pipelines namely U-pipe and V-pipe and each one has its separate ALU.  There  are  five  stages  in  which  these  pipelines  operate,  namely, prefetch,   decode-1,   decode-2,   execute,   writeback.   The   U-pipe   is responsible for executing all integer as well as floating-point instructions while V-pipe executes simple integer and some floating-point instructions. Here, the instruction fetch reads the instruction one at a time and stores them in the instruction queue. During the execution of an instruction, the processor does not sit idle and checks for the next two instructions in the queue. If the two instructions are independent of each other then U-pipe

and  V-pipe  are  assigned  instructions  individually  so  that  execution  can occur simultaneously. However, in the case, the queued instructions are dependent  on  each  other  then  both  the  instructions  are  assigned  to  U-pipe for execution one after the other and V-pipe remains idle. The controlling of the operations of the Pentium processor is provided by the control ROM that has a microcode within it. The control ROM directly controls U-pipe and V-pipe. Both data and code cache within the processor is organized in the 2-way associated set cache. Each cache has 128 sets and each set has 2 lines which  are  32  bytes  wide.  The  LRU  (Least Recently  Used)  mechanism handles the cache replacement. As  we  can  see  clearly  in  the  above  figure  that  the  code  cache  forms  a connection  with  the  prefetch  buffer  by  a  bus  of  size  256  bit,  thus  256/8 i.e., 32 bytes of opcode can be buffered in one clock cycle. The data cache has  two  ports  that  are  used  to  simultaneously  deal  with  two  data references. There  is  an  on-chip  Advanced  Programmable  Interrupt  Controller  that manages interrupt and offers 8259A compatibility.